DEPARTMENT OF ADMINISTRATIVE SERVICES (DAS)

Office of School Construction Grants & Review (OSCG&R)

ROOF KEY PLANS

FORM SCG-3045 INST

INSTRUCTIONS

SCHOOL BUILDING PROJECTS THAT INCLUDE ROOFING

For any project that incorporates new or replacement roof construction, or any project incorporating a total replacement of an existing roof (i.e. down to the roof deck), the Department of Administrative Services, Office of School Construction Grants & Review (OSCG&R) requires that the Local Education Agency (LEA) submit a roof key plan in conjunction with the review / approval process.  The OSCG&R will use the information to prorate the grant calculation for roofs that are between 15 and 20 years old, and when considering roof pitch waivers for existing roofs.

Provide an Overall Roof Key Plan.  Refer to the Sample Roof Plan FORM SCG-3045.  The key plan shall be on one drawing sheet (8 ½” x 11” minimum sheet size) unless there is more than one detached building that is included in the project scope.

The sheet shall be formatted with a title block that includes the name and address of the facility.  Include the current OSCG&R state project number, drawing date, and identify the person creating the document (design professional/company name).  Drawing must include a North arrow.

Use a Legend-style arrangement to provide the following information.  This information must be on a separate sheet; provide the same sheet format described above.

- Each (separate) roof area shall be assigned a designator (A, B, C, D, etc.).  Designators are required for each existing roof area and new roof addition.

- Indicate the square footage for each roof area, and the total roof area for entire building (or buildings).

- Identify the programmatic area(s) immediately under the area of roof replacement.  Use basic descriptions (classrooms, science labs, media center, gymnasium, auditorium, etc.)

- Provide the date of installation for each roof area (if known).  Otherwise leave blank (for future use).

- Identify the roofing material for each area (asphalt shingles, built-up roofing, membrane, etc.). Indicate the roof pitch/slope.

Page 1 of 1