NOTICE TO CONTRACTOR - ENVIRONMENTAL INVESTIGATIONS

Environmental site investigations have been conducted that involved the sampling and laboratory analysis of soil, sediment, concrete, timbers and groundwater collected from various locations and depths within the project limits.  The results of these investigations indicated the presence of detectable concentrations of total petroleum hydrocarbons (TPH), volatile organic compounds (VOC), polynuclear aromatic hydrocarbons (PAH) and RCRA 8 metals in the soils within proposed construction areas.  The DEP groundwater classification beneath the site is GA.  Based on these findings, three (3) AOEC’s exist(s) within the proposed project limits.  In addition, “Low Level” AOEC’s exist within the proposed project limits, where the compounds detected at concentrations below the numeric criteria.  The presence of the compounds at these concentrations will not require material-handling measures beyond those required for normal construction operations.  The presence of these compounds at these concentrations will require the disposition of soils excavated from these areas to be restricted as described herein.  Material excavated from within the “Low Level” AOEC’s that cannot be reused within the Project limits will require disposal at an approved treatment/disposal facility in accordance with Item No. 0202315A - Disposal of Controlled Materials.

The Contractor is hereby notified that controlled materials requiring special management or disposal procedures will be encountered during various construction activities conducted within the project limits.  Therefore, the Contractor will be required to implement appropriate health and safety measures for all construction activities to be performed within the AOEC(s).  These measures shall include, but are not limited to, air monitoring, engineering controls, personal protective equipment and decontamination, equipment decontamination and personnel training.  WORKER HEALTH AND SAFETY PROTOCOLS WHICH ADDRESS POTENTIAL AND/OR ACTUAL RISK OF EXPOSURE TO SITE SPECIFIC HAZARDS IS SOLELY THE RESPONSIBILITY OF THE CONTRACTOR.

The City/Town of XXXX, as Generator, will provide an authorized representative to sign all manifests and waste profile documentation required by disposal facilities for disposal of contaminated sediments, timbers, concrete water, and controlled and hazardous materials.

All suitable material excavated within the “Low Level” AOEC(s) shall be utilized as fill/backfill within the project limits, in accordance with the following conditions: (1) such soil is deemed to be structurally suitable for use as fill by the Engineer; (2) such soil is not placed below the water table; 3) the DEP groundwater classification of the area where the soil is to be reused as fill does not preclude said reuse; and (4) such soil is not placed in an area subject to erosion.  Soils within the “Low Level” AOEC(s) are to be reused on-site prior to the use of other soils and/or fill such that no excess soils requiring off-site disposal are generated from the “Low Level” AOEC(s).

The Sections which shall be reviewed by the Contractor include, but are not limited to, the following:

- Item No. 0101000A - Environmental Health and Safety

- Item No. 0101109A – Hazardous Materials Excavation

- Item No. 0101128A - Securing, Construction and Dismantling of a Waste Stockpile and Treatment Area

- Item No. 0101126A – Disposal of Hazardous Waste

- Item No. 0101130A – Environmental Work – Solidification

- Item No. 0101133A – Disposal of Contaminated Railroad Ties

- Item No. 0101117A - Controlled Materials Handling

- Item No. 0101140A – Disposal of Contaminated Timber Piles

- Item No. 0202315A - Disposal of Controlled Materials

- Item No. 0202318A - Management of Reusable Controlled Material

- Item No. 0202642A – Abandonment of Wells

- Item No. 0202761A – Handling and Disposal of Contaminated Concrete

- Item No. 0204213A – Handling Contaminated Groundwater

The Contractor is alerted to the fact that a Department environmental consultant will be on site for excavation and dewatering activities within the AOEC(s), to collect soil and groundwater samples (if necessary), and to observe site conditions for the State.  The WSA on the plans is to be used exclusively for temporary stockpiling of excavated materials from within project AOEC(s) for determination of disposal classification. Access to the WSA may be limited.  The Contractor shall co-ordinate with [insert name and phone # here] for WSA access.

Information pertaining to the results of the environmental investigations discussed can be found in the documents listed below.  The results contained in the environmental investigation reports listed below show levels of various contaminants that the Contractor may encounter during construction. Actual levels found during construction may vary and such variations will not be considered a change in condition provided the material can still be disposed as non-hazardous at one or more of the disposal facilities listed in Item No. 0202315A - Disposal of Controlled Materials. These documents shall be available for review at the Office of Contracts, 2800 Berlin Turnpike, Newington, Connecticut.

- Task 110 - Corridor Land Use Evaluation. Interchange Modications: Interstate 84 at Route 72, Plainville, Connecticut. Maguire Group Connecticut, Inc. October 28, 1998.

Task 210 - Surficial Site Investigation. New Entrance & Exit Ramps Route 72 Westbound to Route 372.  Plainville, Connecticut.  Maguire Group Inc., April 4, 2000.