ITEM #1019027A - PRE-ASSEMBLED AERIAL CABLE

DESCRIPTION: This work shall consist of furnishing and installing pre-assembled aerial cable, with insulators and brackets, on proposed poles at the location indicated on the plans to maintain illumination circuits.

MATERIALS:  Pre-assembled aerial cable shall be 7 strand aluminum containing a No. 6 AWG bare messenger with three No. 6 AWG cross-linked polyethylene insulated conductors rated at 600 volts.

CONSTRUCTION METHOD:  The pre-assembled aerial cable shall be attached to poles with insulators, including all connections as indicated on the plans or as directed by the Engineer. When necessary, the pre-assembled aerial cable shall be relocated to maintain different illumination circuits as dictated by the construction stages. Pre-assembled aerial cable used for temporary lighting shall be removed once the permanent lighting is installed and operational.  Removed aerial cable shall remain the property of the Contractor.

Pre-assembled aerial cable shall be used to maintain ramp lighting circuits and mainline circuits where the need for large diameter cable is not necessary to maintain proper voltage drop levels.

METHOD OF MEASUREMENT:  This work will be measured for payment by the actual number of linear feet of pre-assembled aerial cable installed and accepted, including attachments.

BASIS OF PAYMENT:  This work will be paid for at the contract unit price per linear foot for "Pre-Assembled Aerial Cable" of the size and voltage specified, complete in place, which price shall include, insulators, entrance cap and attachment, bracket, all materials, tools, connections, equipment, labor, and work incidental thereto.  The unit cost for this item is a one time only cost.  The cost of removing and relocating the aerial cable to maintain different illumination circuits shall be included in the unit cost.