ITEM #1019053A - AERIAL CABLE (3 NO. 2)

DESCRIPTION: This work shall consist of furnishing and installing aerial cable, with insulators and brackets, on proposed poles at the location indicated on the plans.

MATERIALS:  Aerial cable shall be 7 strand aluminum containing a No. 2 AWG bare messenger with three No. 2 AWG cross-linked polyethylene insulated conductors rated at 600 volts.

CONSTRUCTION METHOD:  The aerial cable shall be used as temporary circuitry to maintain nighttime roadway lighting, on the roadways within the project limits open to vehicular traffic. Aerial cable shall be attached to proposed fiberglass poles or aluminum light standards (with insulators), including all connections as indicated on the plans or as directed by the Engineer. When necessary, the aerial cable shall be relocated to maintain different illumination circuits as dictated by the construction stages. Aerial cable used for temporary lighting shall be removed once the permanent lighting is installed and operational.  Removed aerial cable shall remain the property of the Contractor.

Aerial Cable (3 No. 2) shall be used for mainline circuit connections where voltage drop considerations require a large gauge cable.

METHOD OF MEASUREMENT:  This work will be measured for payment by the actual number of linear feet of aerial cable installed and accepted, including attachments.

BASIS OF PAYMENT:  This work will be paid for at the contract unit price per linear foot for "Aerial Cable (3 No. 2)" of the size and voltage specified, complete in place, which price shall include, insulators, entrance cap and attachment, bracket, all materials, tools, connections, equipment, labor, and work incidental thereto.  The unit cost for this item is a one time only cost.  The cost of removing and relocating the aerial cable to maintain different illumination circuits shall be included in the unit cost.