ITEM #1020001A - WOOD POLE

Description:  This work shall consist of furnishing and installing a wood pole with bracket, anchors, ground rod, and conduit, at the location shown on the plans.

Materials:  Wood poles shall conform to the pertinent requirements of Article M.16.04-2. Wood poles shall be 35' in length.

Anchors shall conform to the pertinent requirements of Article M.16.04-2b, c, d, and e.

Brackets shall be fabricated from tubular steel with an upsweep design and shall accommodate a 2" slip-fitter type luminaire. The bracket shall be galvanized and designed for wood pole attachment. Length shall be as called for on the plans.

Ground rods shall be 5/8" x 8' copper-clad steel with an approved square head bolt-type ground clamp.

Ground wire shall be #8 AWG stranded bare soft-drawn copper.

Conduit shall conform to M.15.09.

Construction Method:  The wood pole, with required anchors, bracket, ground rod and ground wire, including excavation of earth or rock, shall be installed as detailed, at the locations shown on the plans.

The actual location of anchor shall be such as to provide adequate support for the bracket being attached.

Method of Measurement:  This work will be measured for payment by the number of wood poles with associated equipment complete and accepted in place.

Basis of Payment:  This work will be paid for at the contract unit price each for "Wood Pole", as specified and complete and accepted in place which price shall include all materials including wood pole, brackets, conduit, anchors, ground rod, ground wire, including excavation of earth or rock and all labor, tools and work incidental thereto.