ITEM#1106001A- 1 Way PEDESTRIAN SIGNAL Pole Mounted

ITEM#1106003A- 1 Way PEDESTRIAN SIGNAL Pedestal Mounted

ITEM#1106004A- 2 Way PEDESTRIAN SIGNAL Pedestal Mounted

Section 11.06 is modified as follows:

Article 11.06.02 Pedestrian Signal, Materials

Article M.16.07 C. Optical Unit

Delete 2. LED: and replace with the following:

General

- Meet requirements of current MUTCD Section 4E.

- Meet current ITE specifications for Pedestrian Traffic Control Signal Indications - (PTCSI) Part 2: Light Emitting Diode (LED).

- Meet CT DOT, 2008 - 2010 Functional Specifications for Traffic Control Equipment; Section 5D, LED Pedestrian Signal with Countdown Timer.

- Meet EPA Energy Star® requirements for LED Pedestrian Signal Modules.

Operational

- Countdown display only during the flashing Pedestrian Clearance (Ped Clr) Interval. Timer goes blank at end of flashing ped clr even if countdown has not reached zero.

Physical

- Sealed optical module to prevent entrance of moisture and dust.

- Self-contained optical module, including necessary power supplies.

- Designed to securely fit into standard housing without the use of special tools or modifications to the housing.

- Identification information on module: manufacturer’s name, model number, serial number, and date code.

Optical

- Multiple LED sources; capable of partial loss of LED’s without loss of symbol or countdown message.

- Two complete self-contained optical systems. One to display the walking person symbol (walk) and the hand symbol (don’t walk). One to display the countdown timer digits.

- Visual Image similar to incandescent display; smooth, non-pixilated.

- Symbol and countdown digit size as shown on the plan.

- Solid hand/person symbol; outline display not allowed.

- Overlaid hand/person symbols and countdown digits arranged side by side.

- Countdown digit display color: Portland Orange in accordance with ITE requirements.

- Countdown digits comprised of two seven segments, each in a figure 8 pattern.

- Photometric Requirements: Luminance, Uniformity, and Distribution in accordance with ITE requirements.

- Color Uniformity in accordance with ITE requirements.

- Blank–Out design; symbols and digits illegible even in direct sunlight when not illuminated.

Electrical

- Operating voltage: 89 VAC to 135 VAC.

- Low Voltage Turn-Off: 35 VAC.

- Turn-On and Turn-Off times in accordance with ITE specifications.

- Combined Hand – Countdown Digits wattage: > 20 Watts.

- Input impedance at 60 Hertz sufficient to satisfy Malfunction Management Unit (MMU) requirements.

- Two separate power supplies. One to power the walking person symbol. One to power the hand symbol and the countdown digits.

- Meet Federal Communication Commission (FCC) regulations concerning electronic noise.

- Filtered and protected against electrical transients and surges.

Warranty

- Five years from date ownership is accepted.