ITEM #0100427A - WATER RESCUE OPERATIONS

Description:  The Contractor shall furnish, maintain and operate one or more water rescue operation boat(s) for Contract work over or adjacent to water.  The boat shall patrol in the vicinity of each above-water work location and be available for water rescue operations.  These water safety measures will be required to protect all Contractor and Department staff.  The Contractor shall also obtain all necessary permits and licenses for the boat and its operators.

Construction Methods: Work under this item shall be performed in accordance with OSHA Article 29 CFR 1926.106 and the following requirements:

- Boat(s) shall be a minimum of twenty (20) feet in length with a stable, flat-bottom and shall be designed specifically for emergency life-rescuing operation.

- Boat(s) shall be equipped with oars and a motor with power within the minimum and maximum horsepower requirements indicated by the manufacturer of the boat(s) provided.

- Each boat shall be equipped with required safety equipment:  life vests, protective clothing, life line, anchor, emergency first aid kit, oxygen equipment and backboard.

- A communication system, such as a walkie-talkie, shall be used to inform the boat operator(s) of an emergency and to inform the operator(s) where the boat is needed.  The operator(s) and at least one worker at each work location over water shall be equipped with a communication device.

- The operator(s) must possess the following current certifications issued by the American Red Cross or equivalent certifications approved by the Engineer:

  - Adult First Aid including CPR training

  - Life Guard Training or Water Rescue Operations

- Boat(s) shall remain in the water when workers are above water and must be capable of being quickly launched to respond to an emergency within three (3) to four (4) minutes.

- There must be at least two (2) rescue operators available on-board each boat when work is being performed over or adjacent to water.

- Each boat shall be operable and available at all times when work is being performed over or adjacent to water.  In the event of a breakdown, above-water work must be discontinued until a boat is repaired or a replacement boat is on Site and in the water.

- The number of boats required must be determined based on the following:

  - The number of work locations where there is a danger of falling into water

  - The distance to each of those locations

  - Water temperature, currents, dams, rapids and other hazards

  - Appropriate response times for rescue

Method of Measurement:  Water Rescue Operations, being paid for on a lump sum basis, will not be measured for payment.  The Contractor shall submit a proposed schedule of values to the Engineer for review and approval.

Basis of Payment:  This work will be paid for at the Contract lump sum price for “Water Rescue Operations,” which price shall include all labor, equipment, materials, maintenance, fuel, repairs, storage and services incidental thereto.

Pay Item	Pay Unit

Water Rescue Operations	l.s.